第四讲 Random Processes——Part II    2025.9.17

Stationary Processes

练习一：判断(True or False)

(   ) Any ergodic random process must be wide-sense stationary.

任何各态历经（遍历）随机过程一定是宽平稳的。

(   ) A strictly stationary random process is always ergodic.

严平稳随机过程一定是各态历经（遍历）的。

(   ) All wide-sense stationary processes are strictly stationary.

中文：所有宽平稳随机过程都是严平稳的。

结论：

练习二：自相关函数的性质

Suppose X(t) is a wide-sense stationary random process. Its auto correlation function is RX(τ)=16+9e−2|τ|

Solve the following questions:

- Find the average power of X(t).

2) Find the DC power of X(t).

3) Find the AC power of X(t).

练习三：Gaussian Random Processes

判断(T or F):

(   )  For a general random process, strict-sense stationary implies wide-sense stationary.

对于一般随机过程，严平稳可以推出宽平稳。

(   )  If two random variables are uncorrelated, they must be statistically independent.

若两个随机变量不相关，则它们一定统计独立。

(   )  For Gaussian random variables, uncorrelated means statistically independent.

对于高斯随机变量，不相关等价于统计独立。

练习四 误码率Pe的计算  Pe=12erfcEbN0

For a BPSK system over AWGN channel, the energy per bit Eb=4×10−6 J, single-sided noise power spectral density N0=1×10−6 W/Hz. Given: erf (2)=0.995322

Calculate the bit error probability Pe.

练习五 Random Processes Passing through Linear Systems

Let X(t) be a stationary stochastic process with mean value a and auto correlation function Rx(τ). Its output after passing through a linear system is  Y(t)=X(t)+X(t−T)

(1) Draw the block diagram of this linear system.

(2) Find the auto correlation function and power spectral density of Y(t).

表1 专业词汇中英文对照

| 中文术语 | 英文术语 |
| --- | --- |
| 严平稳随机过程 | Strict-sense stationary random process |
| 宽平稳随机过程 | Wide-sense stationary random process |
| 各态历经性（遍历性） | Ergodicity |
| 自相关函数 | Autocorrelation function |
| 功率谱密度 | Power spectral density(PSD) |
| 维纳-辛钦定理 | Wiener-Khinchin Relation(Theorem) |
| 高斯随机过程 | Gaussian random process |
| 统计平均 | Statistical average |
| 时间平均 | Time average |
| 统计独立 | Statistically independent |
| 不相关 | Uncorrelated |
| 直流功率 | DC power |
| 交流功率 | AC power |