第六讲 Channel——Part I    2025.9.22

Example1:Assume the transmitted power PT=10 W, transmitting antenna gain GT=100, receiving antenna gain GR=10. The propagation distance is 50 km, and the electromagnetic wave frequency is 800 MHz. Calculate the received power and propagation loss.

Example 2: A binary symmetric memoryless channel (BSC). Bit 0 and bit 1 are transmitted with equal probability. The error transition probabilities are P(1|0)=P(0|1)=0.005. Calculate the average bit error probability Pe.

True & False

(     ) 1. The modulation channel is located between the output of the modulator and the input of the demodulator.

(     ) 2. The coding channel mainly analyzes waveform distortion and amplitude-frequency distortion of analog signals.

(     ) 3. The performance of the coding channel depends on the noise and distortion from the modulation channel.

Short answer question:

What is the difference between modulation channel and coding channel?

Example 3 : A two-path multi-path channel, the channel transfer function is

H(ω)=Ae−jωτ01+e−jωτ The relative time delay between two paths is τ=2 μs.

- Find the frequencies where deep fading (nulls零点) happens.

- Calculate the coherence bandwidth of this two-ray channel.

| No. | English | 中文 |
| --- | --- | --- |
| 1 | attenuation | 衰减 |
| 2 | propagation | 传播 |
| 3 | symmetrical cable | 对称电缆 |
| 4 | coaxial cable | 同轴电缆 |
| 5 | optical fiber | 光纤 |
| 6 | ground wave | 地波 |
| 7 | sky wave | 天波 |
| 8 | LOS (line of sight) | 视距传播 |
| 9 | constant parameter channel | 恒参信道 |
| 10 | random parameter channel | 随参信道 |
| 11 | frequency selective fading | 频率选择性衰落 |

表1 专业词汇中英文对照