学号：                   姓名：

第二章 课后作业

1.1 An information source consists of A, B, C and D. These symbols are represented by codewords 00, 01, 10 and 11. If each binary symbol is transmitted by the pulse with 5ms, then find the average information rates respectively under the following conditions:

(1) The four symbols have equal probability of occurrence (equal probability).

- The four symbols have the following probabilities of occurrence.

1.2 Assume a signal source produces 4-ary signals with equal probability, and the width of symbol is 125μs. Find its symbol rate and information rate.

1.3 Assume the information rate is 2400b/s of a 4-ary digital transmission system (equal probability). During half an hour, the number of the received symbols in error is 216. Try to find the symbol error probability Pe.