学号：   写群里名单的小学号                姓名：

第二章 课后作业

2. 1   Let the stochastic process ξ(t) be expressed as   ξ(t)=2cos(2πt+θ)

where θ is a discrete random variable. P(θ=0)=1/2, Pθ=π2=1/2.

Compute:(1) The mean value Eξξ(1)

(2) The auto-correlation function Rξ(0,1)

--Formula reminder for discrete random variable expectation: EX(t)=i=1KxiP(xi)

2.2: Let X1 and X2 be statistically independent Gaussian-distributed random variables, each with zero mean and variance σ2. Define the Gaussian process

X(t)=X1cos(ω0t)+X2sin(ω0t)

Find the mean value μX(t) and auto-correlation function RX(t1,t2).

Given Conditions:

- E[X1]=0, E[X2]=0

- Variance: D(X1)=σ2, D(X2)=σ2

- X1 and X2 are independent ⇒E[X1X2]=E[X1]E[X2]=0

- E[X12]=D(X1)+(E[X1])2=σ2, E[X22]=σ2

2.3: Let s(t) be a stationary random pulse sequence with power spectral density Ps(f). Find the power spectral density Pe(f) of the modulated signal e(t)=s(t)cosω0t.

- 2.4 Let X(t) be a stationary stochastic process with mean value a and auto correlation function Rx(τ). Its output after passing through a linear system is Y(t)=X(t)+X(t−T)

- (1) Draw the block diagram of this linear system.

- (2) Find the auto correlation function and power spectral density of Y(t).

2.5 A zero-mean Gaussian white noise with power spectral density n02 is fed into an ideal band pass filter. The filter has a center frequency fc and bandwidth B, as shown in the figure.(1) Find the auto-correlation function of the filter’s output noise.

(2) Write down the one-dimensional probability density function of the output noise.

2.6A two-path multi-path channel, the channel transfer function is

H(ω)=Ae−jωτ01+e−jωτ The relative time delay between two paths is τ=2 μs.

- Find the frequencies where deep fading (nulls零点) happens.

- Calculate the coherence bandwidth of this two-ray channel.

- 2.7 思考题(从时域和频域两个域进行推导)