- 1.3信息及其度量 &  1.4通信系统性能指标​

- 1.3信息及其度量

Example 1：there is a binary discrete source, if p(0)=p (1)=1/2, then

I(0)=      ;      I(1)=

Example 2: A source has three symbols A, B, C with probabilities:

p(A)=1/2, p(B)=1/4, p(C)=1/4;

A 10-symbol message: A A B C A B A C A A. Find the total information of this message.

Example 3: A discrete source has four possible symbols x1,x2,x3,x4 with probabilities:   p(x1)=1/4, p(x2)=1/8,p(x3)=1/8, p(x4)=1/2

Find the entropy of this source (in bits per symbol)

- 1.4通信系统性能指标​

Example 1:  In a digital communication system, the symbol duration is T=0.2 ms. The system adopts octal transmission (each symbol carries 3 bits of information).

- ①Calculate the symbol rate RB (Baud).

- ②Calculate the information bit rate Rb (bit/s).

- Solution

- Symbol rate:

- RB=1T=10.2×10−3=5000 Baud

2  For octal M=8,log28=3

- Rb=RB⋅log2M=5000×3=15000 b/s

出门测(2024期末) Suppose that an information source is composed of 4 different symbols, with the probabilities of 1/4, 1/8, 1/8 and 1/2. If the source sends out 2400 independent symbols per second, try to find the average information rate of the information source.